Supporting Information for title of study: Assessing the ecological impacts of NORM-contaminated scale on marine infauna using sediment microcosms

This supporting information contains supporting methodological details, figures and tables.

Table of Contents

1	Supporting Methodology:	3

1.1	Preparation and radiochemical analysis of barite scale and overlying seawaters	3

1.2	Creation of sediment mixture and physical chemical properties	4

1.3	Sediment total organic carbon (%) analyses	5

1.4	Preliminary radiological biota dose assessment using the ERICA Tool	7

1.5	Analytical methods for metals; overlying waters, sediments and biota	8

1.6	Alpha spectrometry for 226Ra and 210Po of sediment and biota tissues	9

1.7	Lead-210 by beta spectrometry via liquid scintillation counting of sediment and biota tissues	11

2	Supporting Figures	13

Supporting Figure 1	13

Supporting Figure 2	14

Supporting Figure 3	16

Supporting Figure 4	17

3	Supporting Tables	20

Supporting Table 1	20

Supporting Table 2	22

Supporting Table 3	23

Supporting Table 5	24

Supporting Table 6	25

Supporting Table 7	27

Supporting Table 8	29

Supporting Table 9	31

Supporting Table 10	32

Supporting Table 11	33

Supporting Table 12	35

# Supporting Methodology:

## Preparation and radiochemical analysis of barite scale and overlying seawaters

Scale material was collected in bulk and approximately 30 g (d.w.) was weighed and mechanically pulverised to <30 µm of homogenised material. A sub sample of 5 g (d.w.) was analysed via alpha and gamma spectrometry to determine the presence of radionuclides present in the 238U, 235U and 232Th decay chains according to certified standard methods (ANSTO Minerals methods AM-I-052-003 Preparation of Powdered Samples for Gamma Ray Analysis, AM-I-052-004 Counting Procedure using Maestro and AM-I-005 Gamma Spectrum Analysis of Solid Samples using GammaVision).

Gamma spectrometry was carried out using EG&G Ortec® HPGe n-type detectors/MCA systems according to established procedures.  Detectors had relative efficiencies of 20–60% and resolution (full width half maximum, FWHM) of 1.8 to 2.0 keV at 1332 keV 60Co. A Maestro®-32 MCA Emulator was used to acquire sample spectra, and Ortec® GammaVision was used to analyse the spectra. Energy calibration was carried out using a standard 226Ra point source. Counting geometries were measured in sealed 500 mL Marinelli beakers with a sample volume of 450 mL.  CANMET certified reference materials, BL-5 and OKA-2, were used to determine liquor efficiency calibrations.  All samples were left for three weeks to allow for 222Rn ingrowth and then gamma counted for two days.

The gamma peaks used in the analyses were: 210Pb (46.50 keV), 230Th (67.8 keV), 235U (205.31 keV) and 231Pa (302.67 keV).  The concentration of 226Ra was inferred from its gamma emitting daughter peaks, 214Pb (351.90 keV) and 214Bi (609.32 keV).  The concentration of 227Ac was inferred from its gamma emitting daughter peak, 227Th- (236.00 keV).  The concentrations of 228Ra and 228Th were inferred from their respective gamma emitting daughter peaks, 228Ac (911.07 keV) and 212Pb (238.63 keV).

Alpha spectrometry was used to determine the concentrations of 210Po according to I-4303 Radiochemical Analysis for U, Th, Pb and Po.  Polonium-209 radioactive tracer solution was used to monitor losses throughout the analyses. Samples were analysed ‘as received’ by taking an appropriate sample aliquot.  Impurity elements were removed using a series of precipitation and solvent extraction steps.  Polonium-210 was then autodeposited onto a silver disc prior to alpha counting.  Samples were alpha counted for up to 200,000 seconds using silicon surface barrier detectors and Maestro®-32 MCA Emulator software to obtain sample spectra.  The spectra were then analysed using Ortec® AlphaVision.

The same subsample was also analysed via X-ray fluorescence (XRF) to determine the major elements in the scale and to confirm previous XRD analyses on the same scale material.

## Creation of sediment mixture and physical chemical properties

For this study, a homogenised mixture was created to achieve a sediment that is representative of the low silt: sand ratio and low organic carbon content (TOC; ~4 %) of continental shelf marine sediments surrounding oil and gas installations (Heap and Sbaffi 2008, Radke et al. 2017). To ensure the tolerance of organisms in the sediment mixture, a survival test was performed using polychaetes and was determined as the percent survival (%) of organisms over a period of 3-days. A mean survival rate of 85 % in the sediment was set as the minimum to validate the sediment mixture was appropriate for the scale-spiked exposure test. Two different mixtures were created by mixing ratios of Cronulla sand (C): Bonnet Bay silt (BB) to reach a 10 cm depth into a 1.5 L glass beaker. The two trials were as follows: a) 90% C: 10% BB and b) 92% C: 8% BB. Each mixture was homogenised by placing each into a clean plastic tray and adding small amounts of filtered seawater to produce a slurry. The mixture was constantly turned until the muddy sediment had settled with the sand. Each individual mixture was transferred with a clean spade into a 1.5 L glass beaker and labelled. Overlying seawater (1 µm filtered) was added and monitored daily to ensure appropriate salinity (29 ± 2%), dissolved oxygen levels (85-90 %). After 1-day of sediment settlement, five polychaetes were measured and added to each beaker. At the completion of the survival test, the sediment was gently sieved through a 0.5 mm mesh and the polychaetes retained on the mesh were counted into polyethylene containers with both trials having 100% survival rate. The chosen mixture for the scale-spiked sediment was sandy sediment mixed with muddy sediment at a ratio of 10:1.

The physicochemical properties and concentration of major metal contaminants, as total recoverable metals (TRM), for the reference sediment used in this study are summarised in Supporting Table 2. Characteristics included fine grain size, poor mixing of the fines and silt, and very low total organic carbon (TOC; average of 1.7 % ± 0.51) with an average moisture content of 30.8 % ± 2.53 (Supporting Table 2). Based on comparison of concentrations with the sediment quality guideline (SQG) and sediment quality high (SQG-high) values, all metals did not exceed their respective SQG or SQG-High value and were therefore non-contaminated prior to the experiment.

## Sediment total organic carbon (%) analyses

Following the creation of the reference sediment, three subsamples of the homogenised mixture were collected, weighed and then dried at 40ºC for 12 hours to remove any moisture content and weighed again to retrieve the dry weight.

Samples were homogenised using a ceramic mortar and pestle, then ~5 µg of each subsample was extracted and put into a clean vial for instrumental analysis for TOC. The crushed and dried samples were weighed into tin capsules and introduced sequentially into an elemental analyser (Thermo Fisher Flash 2000 HT EA) using an autosampler. Each sample was combusted into CO2 and N2 in a combustion furnace (silvered cobaltous/ic oxide, chromium oxide, quartz chips and quartz wool) at 1020ºC before being transferred with a helium carrier gas (100 mL/min) into a reduction furnace (copper) at 600 ºC, where any excess nitrous oxides are converted into N2 and excess O2 is removed. The analyte gases passed through a water trap before the CO2 and N2 are separated by a GC column at 40ºC. The gases were then transferred to a Thermo Fisher Conflo IV and into a Thermo Fisher Delta V Plus isotope ratio mass spectrometer for δ13C and δ15N measurements.

The data reported relative to IAEA secondary standards that have been certified relative to VPDB for carbon and air for nitrogen.  A two-point calibration is employed to normalise the data, utilising standards that bracket the samples being analysed.  Two quality control references were also included in each run: IAEA USGS-40 L-Glutamic Acid, USGS-41a L-Glutamic Acid and Elemental Microanalysis B2045 Methionine. Results are accurate to 1% for both N % and C % with a standard deviation of 0.3 ‰ for δ15 N and δ13 C (n=19). Results were expressed in δ notation in parts per mil (‰) and converted to a percentage total:

δ13C=δ(13C/12C)P/reference=R13C/12CP–R13C/12CreferenceR13C/12Creference

where Rsample and Rreference are the isotopic 13C/12C ratios of the sample and reference, respectively.

## Preliminary radiological biota dose assessment using the ERICA Tool

To calculate the appropriate percentage of scale (dry mass) to sediment ratio for the scale-spiked sediments, a preliminary ERICA biota dose assessment was performed to determine the estimated dose rate to reach 10 µGy/hr to a polychaete worm. Four ERICA assessments were performed using various dilution series of the bulk activity concentrations of the main radionuclides (226Ra, 210Po, 210Pb, 228Th) of the neat scale to identify the percentage of scale (10%, 1% and 0.1%) that would result in a dose rate above the screening value of 10 μGy/hr to the most exposed organism i.e. polychaete worm (Supporting Table 3).  Each assessment was run at the Tier 2 stage using a marine ecosystem. Default occupancy factors, concentration ratios (CR) percentage dry mass of sediments, uncertainty factors, and radiation weighting factors were used and are considered conservative.

For each ERICA assessment, total concentrations of the radionuclides in the barite scale (Table 1) were used to estimate absorbed weighted dose rates to marine organisms. These were entered as ‘sediment’ activity concentrations. Corresponding water concentrations were calculated using scale-specific partitioning coefficients (Kd values) for each radionuclide. The Kd values were determined by a seawater leach experiment where various scale samples were continuously rolled in seawater for 1 month in the laboratory (as per MacIntosh et al., 2022). These are scale-specific Kd values to the barium sulphate scale used in this study.

The estimated total dose rates for the polychaete worm exposed to sediment activity concentrations of 1-100 % scale were within the ICRP 400-4000 μGy/hr dose band for reference crabs (ICRP, 2017). This suggests activity concentrations of that magnitude may have potential impacts to exposed invertebrates. As the total dose rate for the 0.1 % scale was still above the 10 μGy/hr, but lower than the DCRL band of 400-4000 μGy/hr, it was concluded 0.1% of the total sediment mass containing scale resulted in an estimated dose rate of 10 μGy/hr to a polychaete worm and would provide a non-lethal dose at a relevant rate for the organisms being targeted in this experiment.

## Analytical methods for metals; overlying waters, sediments and biota

Overlying water samples were analysed for dissolved metals by filtering (0.45 µm) and acidifying to 2 % HNO3 (v/v, Tracepur, Merck) and stored at 4°C at CSIRO Land and Water until being analysed. The analyses of waters were using inductively coupled plasma mass spectrometry (ICP-MS) and cold vapour- atomic fluorescence spectrometry (CV-AFS). For QA/QC, triplicate blanks, replicates for 20% of samples, and analyte sample-spikes with certified reference materials (CRM) were analysed. Replicates were within 20% and the recoveries of metals in the spikes and CRM (CASS-6, Canadian Reference Material) were within 91-106 % of expected values. The limits of detection (calculated as 3*SD of the blanks) were less than 10 % of the lowest measured values.

Analyses of total recoverable metals (TRM) were determined as described in Belzunce-Segarra et al. (2015). In brief, all sediment samples were dried 60°C for 12 hr and crushed before the TRM were determined after low-pressure microwave-assisted aqua regia digestion (MARS 5, CEM). Metal analyses were performed using ICP-MS and inductively coupled plasma – atomic emission spectrometer (ICP-AES). Analyses of digest blanks, replicates for 20% of samples, analytic sample-spikes and the CRM (AGAL-10, Australian Government National Measurement Institute) were undertaken for QA/QC. Recoveries were within 85-97% of expected values and the limits of reporting were less than 10% of the lowest reported values.

Tissues were freeze-dried and weighed before being separately placed into clean tetrafluorometoxil (TFM) digestion tubes for reverse aqua regia microwave digestion.  10 mL of reagent grade HNO3 (Analar, Merck, Darmstadt, Germany) was added to each tube and capped. Tubes were left at room temperature for 18 h to pre-digest.  Vials were subsequently heated in a microwave oven (MARS Express) at 200°C for 20 minutes (1600 W; 100% power) and then left at room temperature to cool for 30 min.  Samples were transferred to 10 mL glass vials via pipette and each polycarbonate vial was rinsed with MilliQ water, with rinsings transferred to a glass vial for a final volume of 5 mL. Trace metals were analysed for all acid digest samples via inductively coupled plasma mass spectrometry (ICP-MS; Agilent 8800) and mercury was analysed via cold vapour atomic fluorescence spectrometry (CV-AFS).  For quality control, analyses of acid-digest blanks, replicates for 20% of samples, spiked-samples and CRMs (DOLT-4 Dogfish liver, National Research Council of Canada, NRCC; DORM-3 Fish protein, NRCC) were performed. Replicates were within 20 % of each other and recoveries for spikes and CRMs were within 86-110 % of expected values.

## Alpha spectrometry for 226Ra and 210Po of sediment and biota tissues

210Po in the sediment and biota tissues were analysed by alpha spectrometry following the ERMC Polonium chemical isolation method. After the sediment and biota tissues were acid-digested, subsamples of the digest solutions (50 mL) were divided, and 20 mL was allocated for 210Po.  The dissolved solutions were dissolved in 80 mL of 0.04 M HCl, then treated with 0.2 g ascorbic acid to reduce Fe (III) to Fe (II). 100 uL 1.0 M citric acid solution was added to complex trace iron and chromium and 10 mg Bi3+ holdback carrier was added to inhibit autodeposition of bismuth. The pH of each sample was adjusted to 1.5 with the addition of cresol red indicator and NH4OH. 1 g of hydroxylammonium chloride was added to each solution and then polonium was deposited at 80°C and pH 1.5-2 on a 17 mm silver disc, attached to a SpinFin as a disc holder. The disc was immersed for 5 h with continuous stirring at 400 rpm. Silver disks were removed and rinsed in distilled water and ethanol prior to counting via alpha spectrometry for up to 200,000 s using silicon surface barrier detectors and Maestro®-32 MCA Emulator software to obtain sample spectra.  210Po activities were calculated from the peak counts of 210Po and 209Po. The resulting spectra were then analysed using Ortec® AlphaVision.

Following the separation of 210Po, the ERMC Radium chemical isolation procedure was applied to all the samples. Samples were added to 800 mL Milli-Q water and 20 mL conc. H2SO4. 10 Ml 10 mg/mL Pb2+ carrier was slowly added via burette while the sample was stirred. Samples were covered and left overnight to allow the Pb/Ba/Ra sulphate precipitate to flocculate. The supernatant was discarded with the white precipitate being transferred to clean 50 mL centrifuge tubes and centrifuged for 5 mins at 4500 rpm. 5 mL of 0.2 M Na5DTPA was added to the remaining residue along with thymol blue indicator to verify a sample pH of >9. Samples were mixed via vortex mixer and placed in a sonicator bath for 30 mins. After the addition of two drops of methyl red indicator, samples were passed through a 0.45 µm disposable membrane filter. 2 mL of 1:1 acetic acid/water and 1 mL BaSO4 seeding suspension (sonicated for 15 minutes prior) were simultaneously added to each sample before samples were refrigerated for at least 30 minutes. Refrigerated samples were poured through a smooth-surfaced Millipore “VV” membrane filter in a lock-seal Gelman filter apparatus and allowed to drain. Membrane filters were removed and left for at least three weeks to allow for radon-222 (222Rn) ingrowth prior to counting. Sources were counted for at least 21 h using the HPGe detectors. The activity of 226Ra was determined by the average activity of 214Pb (using energy line 351.9 keV) and 214Bi (using energy line 609.3 keV).

## Lead-210 by beta spectrometry via liquid scintillation counting of sediment and biota tissues

210Pb was measured by beta spectrometry using a Ludlum Model 43-10 instrument according to modified in-house methods and manufacturer instructions (ERMC I-3327 Lead-210 Analysis by Beta Spectrometry).

For the analysis of 210Pb, 20 mL of the acid-digested solutions were placed on a hot plate stirrer. 1 mL of concentrated acetic acid (CH3COOH) to each sample followed by 1 drop of methyl orange (solution turns red). The pH was adjusted to be ~3.7 with 2 mL of concentrated NH3 (indicator turns light orange). 5 mL of 20% sodium chromate (Na2CrO4) was slowly added to produce a yellow mixed precipitate lead chromate (Pb(Bi)CrO4). Samples were left on the hot stirrer until the precipitate coagulated (at least 10 mins). Once the precipitate formed, the beakers were removed from the hot plate to cool. Collection of the precipitate was through a pump filtration unit fitted with a nitrate membrane filter (Millipore Type GS, 0.22 µm, 25 mm). A few drops of Milli-Q water were added to the filter to wet the surface. The vacuum pump was then turned on. Each sample solution was poured into the funnel of the filtration unit, with the vacuum pump valve being adjusted to prevent the sample from being filtered too quickly. Once the precipitate was collected onto the filter paper, the filter paper was carefully removed from the filtration unit using a pair of clean tweezers. The filter paper with the precipitate was transferred to a beta counting planchet and allowed to dry. The 210Pb/210Bi source was counted after four weeks to allow for the complete ingrowth of 210Bi and the decay of 212Pb and 212Bi. Beta counting was by measuring the activity peak of 210Bi (1.16 MeV). Samples and standards were counted for 100-min cycles for at least three cycles. After counting, the membrane filter beta source plus sticky tape was transferred into a 100 mL beaker. 5 mL of conc. HNO3 was added to each sample, covered with a watch glass and refluxed on the hot plate until the precipitate completely dissolves. After cooling, the digest solution was transferred into a 100 mL volumetric flask and filtered with water and made up to 100 mL with water. 10 mL of the solution was transferred into a vial (labelled with the sample ID) and sent for stable Pb analysis via ICP-AES to obtain the Pb chemical yield.

# Supporting Figures

Supporting Figure 1 X-ray diffraction (XRD) results of pulverised scale (<30 mm) collected from subsea well tubular pipes off the coast of Western Australia. All peaks in red are those identified as barite (BaSO4).

Supporting Figure 2 Total recoverable metals (TRM) and acid-extractable metals (AEM; 1 HCl; 1 hr) in the sediments after 28 days of exposure Control (orange) is indicative of control sediments with no scale, Surface (blue) represents scale added to the surficial layer to a final concentration of 0.1% relative to the sediment mass and Mixed (red) represents scale mixed with the sediment to a final concentration of 0.1% relative to the sediment mass. Significant differences between treatment groups are signified by ‘*’ for p < 0.05 and ‘**’ for p < 0.01. Relative fraction of elements found to be acid extractable as a proportion of the total elemental concentration are shown by their percentage values (%) on each column. None of the TRM and AEM exceeded the ANZG default guideline values for sediments.

Supporting Figure 3 Time-averaged dissolved (0.45 µm) metals in overlying seawater over the 28 days (mean +/- SE, n=3 per day per treatment) for each day of sampling. Control (yellow) is indicative of control sediments with no scale, Surface (blue) represents scale added to the surficial layer to a final concentration of 0.1% relative to the sediment mass and Mixed (red) represents scale mixed with the sediment to a final concentration of 0.1% relative to the sediment mass.

Supporting Figure 4 Individual radionuclide contributions to the internal dose rates (µGy/hr) to the exposed marine sediment organisms in (a) scale-mixed treatment group and (b) scale-surface treatment group. Dose rates were calculated from the scale-treatment specific activity concentrations (sediment, overlying seawater, experimental- specific Kd values, whole-body activity concentrations in V. australiensis). All details and assumptions are provided in Section 2.10. The ERICA default 10 µGy/hr screening value and the IRCP Derived Consideration Reference Level for a crab are not shown, due to the low external dose rates

# Supporting Tables

Supporting Table 1 X-ray fluorescence (XRF) results of the pulverised (30 µm) barite scale. Results are expressed as a percentage (%). The main elements in the pulverised scale are highlighted in bold red.

| Percentage of each element detected by X-ray fluorescence (XRF) in the pulverised scale |  |  |  |  |
| --- | --- | --- | --- | --- |
| Al | As | Ba | Ca | Ce |
| <0.01 | 0.002 | 56.9 | 0.069 | 0.012 |
| Co | Cr | Cs | Cu | Dy |
| <0.001 | 0.006 | 0.008 | <0.001 | <0.010 |
| Er | Eu | Fe | Gd | Hf |
| <0.010 | <0.010 | 0.31 | <0.010 | <0.001 |
| Ho | K | La | Lu | Mg |
| 0.016 | 0.006 | 0.052 | <0.010 | <0.01 |
| Mn | Mo | Na | Nb | Nd |
| <0.001 | <0.001 | 0.17 | 0.006 | <0.010 |
| Ni | P | Pb | Pr | Rb |
| <0.001 | 0.002 | 0.01 | <0.010 | <0.001 |
| S | Sc | Si | Sm | Sn |
| 13.8 | 0.013 | 0.06 | <0.010 | 0.019 |
| Sn | Sr | Ta | Tb | Th |
| 0.019 | 0.864 | <0.001 | 0.013 | 0.002 |
| Ti | Tm | U | V | Y |
| 0.004 | <0.010 | 0.009 | 0.04 | 0.013 |
| Yb | Zn | Zr |  |  |
| <0.010 | <0.001 | 0.019 |  |  |

Supporting Table 2 Physiochemical properties of the initial reference sediment (90% Cronulla sand: 10% Bonnet Bay silt) used for the 28-day exposure experiment to represent continental shelf marine sediments. Data is presented as mean ± SE (n=3; 2 s.f.). Sediment Guideline Value (DGV) and GV-High are obtained from the revised ANZECC/ARMCANZ sediment quality guidelines as per ANZG (2018). No known sediment guideline values for metals are represented by ‘-‘. Data for TOC are derived from triplicate samples of the initial sediment.

|  |  |  |  |  | Total recoverable metals (TRM; mg kg-1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | As | Cd | Co |  | Cr | Cu |  | Mn | Ni | Th | U | Zn | Hg | Ba | Ca | Fe | Pb | TOC (%) |  | Moisture content (%) |
| Reference sediment |  | 0.8 ± 0.2 | 0.01 ± 0 | 0.2 ± 0 |  | 2.2 ± 0.2 |  | 1.4 ± 0.1 | 5.6 ± 1.1 | 1.8 ± 0.4 | 0.6 ± 0.1 | 0.2 ± 0.04 | 5.3 ± 0.2 | <0.02 | 81760 ± 16270 | 81760 ± 16270 | 1820 ± 1080 | 4.9 ± 2.6 | 1.7 % ± 0.5 |  | 30.8 % ± 2.5 |
| ANZG (DGV) |  | 20 | 1.5 | - |  | 80 |  | 65 | - | 21 | - | - | 200 | 0.15 | - | - | - | 50 |  |  |  |
| ANZG GV- high |  | 70 | 10 | - |  | 370 |  | 270 | - | 52 | - | - | 410 | 1 | - | - | - | 220 |  |  |  |

Supporting Table 3 Output from the ERICA biota dose assessments to determine the non-lethal concentration of pulverised barite scale (dry mass) by total sediment mass exposed to a polychaete worm that would result in a total dose rate equivalent to the screening level of 10 µGy hr. Estimated predicted internal, external and total dose rates (µGy hr-1; 2 significant figures) are illustrated for the polychaete worm reference organism. Scale-specific Kd values from MacIntosh et al., (2022) and ERICA calculated seawater concentrations were used as inputs. Values highlighted in bold exceed the ERICA screening value of 10 µGy/hr and the respective ICRP Derived Consideration Reference Levels (DCRLs) for representative aquatic organisms; 40- 400 µGy/hr for a Reference Flatfish (fish) and Reference brown seaweed (macroalgae) and 400- 4000 µGy/hr for Reference Crab (crustaceans)

|  | Predicted internal dose rate (mGy/hr) |  |  |  |  | Predicted external dose rate (mGy/hr) |  |  |  |  | Total dose rate (mGy/hr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Scale contribution in sediment | 226Ra | 210Po | 210Pb | 228Th | Total internal dose | 226Ra | 210Po | 210Pb | 228Th | Total external dose | 226Ra | 210Po | 210Pb | 228Th |  |
| 100% scale | 2.3 | 615 | 0.7 | 2.5 | 620.8 | 198 | 3.07E-4 | 1.4 | 61.7 | 261.2 | 200 | 615 | 2.1 | 64.2 | 882 |
| 10% scale | 2.3 | 615 | 0.7 | 0.2 | 618.6 | 19.8 | 3.07E-5 | 0.1 | 6.2 | 26.1 | 22 | 615 | 0.8 | 6.4 | 644.7 |
| 1% scale | 2.3 | 615 | 0.7 | 0.02 | 618.4 | 2 | 3.07E-6 | 1.4E-2 | 0.6 | 2.6 | 4.2 | 615 | 0.7 | 0.6 | 620.9 |
| 0.1% scale | 0.2 | 61.5 | 0.1 | 0.002 | 61.8 | 0.2 | 3.07E-7 | 1.4E-3 | 0.1 | 0.3 | 0.4 | 61.5 | 0.1 | 0.1 | 62.1 |

Supporting Table 5 Details (geometries, occupancy factors, Chi value) about the reference organisms used from the ERICA database that were representative of the test organisms used in the experiment.

| Representative organism from ERICA database | Geometry |  |  |  | Chi value | Occupancy Factor |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Mass (kg) | Length (cm) | Height (cm) | Width (cm) |  | In interior water | At sediment-water surface | In sediment |
| Victoripisa australiensis (amphipod) |  |  |  |  |  |  |  |  |
| Crustacean (ICRP Crab) | 7.54E-01 | 2.00E+01 | 1.20E+01 | 6.00E+00 | 0.30 | 0 | 0.5 | 0.5 |
| Tellina deltoidalis (clam) |  |  |  |  |  |  |  |  |
| Mollusc - bivalve (FASSET Benthic mollusc) | 1.64E-02 | 5.00E+00 | 2.50E+00 | 2.50E+00 | 0.50 | 0 | 0.3 | 0.7 |
| Nephyts australiensis (polychaete) |  |  |  |  |  |  |  |  |
| Polychaete worm (FASSET Benthic worm) | 1.73E-02 | 2.30E+01 | 1.20E+00 | 1.20E+00 | 0.05 | 0 | 0 | 1 |

Supporting Table 6 Concentration ratios (CR) calculated for each of the exposed test organisms in the 28-day experiment for the main radionuclides of interest in the barite scale (226Ra, 210Pb, 210Po, 228Ra, 228Th). CRs for 226Ra and 210Po for the amphipod were calculated using the experimental-specific parameters. CRs for the clam and polychaete were retrieved from the default ERICA CRs (see “*”and “**” for further information).  The CRs for the radionuclides not analysed (Pb, Th, Bi, At, Ac, Tl) were replaced by the default ERICA CRs.

| Organism | Radionuclide | Concentration ratio (CR) for each scale-treatment group |  |
| --- | --- | --- | --- |
|  |  | Surface | Mixed |
| Victoripisa australiensis (amphipod) | Ra* | 1.18E+01 | 3.61E+00 |
|  | Po* | 5.87E+00 | 1.27E+00 |
|  | Pb | 2.03E+04 | 2.03E+04 |
|  | Th | 7.40E+02 | 7.40E+02 |
|  | Bi | 7.60E+01 | 7.60E+01 |
|  | At | 3.92E+01 | 3.92E+01 |
|  | Ac | 7.40E+02 | 7.40E+02 |
|  | Tl | 1.10E+02 | 1.10E+02 |
| Tellina deltoidalis (clam)** | Ra | 5.71E+01 | 5.71E+01 |
|  | Po | 5.72E+04 | 5.72E+04 |
|  | Pb | 6.55E+02 | 6.55E+02 |
|  | Th | 7.60E+01 | 7.60E+01 |
|  | Bi | 5.37E+01 | 5.37E+01 |
|  | At | 7.33E+02 | 7.33E+02 |
|  | Ac | 1.10E+02 | 1.10E+02 |
|  | Tl | 6.55E+02 | 6.55E+02 |
| Nephyts australiensis (polychaete)*** | Ra | 4.70E+02 | 4.70E+02 |
|  | Po | 4.77E+05 | 4.77E+05 |
|  | Pb | 6.55E+02 | 6.55E+02 |
|  | Th | 7.60E+01 | 7.60E+01 |
|  | Bi | 5.37E+01 | 5.37E+01 |
|  | At | 9.91E+02 | 9.91E+02 |
|  | Ac | 1.10E+02 | 1.10E+02 |
|  | Tl | 6.55E+02 | 6.55E+02 |

*Values were calculated from the maximum activity concentration (fresh weight; Bq/kg) in the amphipods from the respective scale-treatment group divided by the highest radionuclide activity concentration in the respective scale-spiked sediments (dry mass; Bq/kg)

** Default values from the ERICA Wildlife Transfer Database (WTD); Ra: WTD2020 Mollusc-bivalve; Po: WTD2020 Mollusc-bivalve; Pb: WTD2020 Mollusc-bivalve; Th: WTD2020 Mollusc-bivalve Surrogate-Th Crustacean; Bi: WTD2020 Mollusc-bivalve; At: Assumes Mollusc-bivalve for Iodine; Ac: Assumes Mollusc-bivalve for Th; Tl: Assumes Crustacean Tl

*** Default values from the ERICA database; Ra: Assumes Crustacean barium; Po: WTD2020 Polychaete worm; Pb: WTD2020 Polychaete worm; Th: Assumes Crustacean Th; Bi: WTD2020 Polychaete worm; At: Assumes Mollusc- bivalve; Ac: Assumes Polychaete worm Pu; Tl: Assumes Crustacean Tl

Supporting Table 7 Data parameterisation inputs for the ERICA assessment assessing the dose rates from the NORM detected in the barite scale from the 28 d experiment, with experimental-retrieved activity concentrations and Kd values.

|  | Radionuclide | Activity concentrations in scale-contaminated sediment (Bq/kg dry mass) |  | Activity concentrations in overlying seawater (Bq/L)d |  | Experiment-retrieved Kde |  | Activity concentrations in whole-body of V. australiensis (Bq/kg)f |  | Calculated activity concentrations in Mollusc- bivalveg |  | Calculated activity concentrations in Polychaeteg |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Mixed | Surface | Mixed | Surface | Mixed | Surface | Mixed | Surface | Mixed | Surface | Mixed | Surface |
| Long-lived radionuclides (> 10 day λ) | Ra-226a | 140 b | 84 b | 9.10E-03 | 6.00E-02 | 3.26E-04 | 4.29E-04 | 6.64E+02 | 1.66E+03 | 3.43E+00 | 3.43E+00 | 2.82E+01 | 2.82E+01 |
|  | Po-210a | 67 b | 71.8 b | 4.00E-01c | 9.10E-03c | 1.27E-01 | 1.36E-04 | 9.10E+01 | 3.93E+02 | 4.28E+02 | 4.28E+02 | 3.56E+03 | 3.56E+03 |
|  | Pb-210 | 65 | 65 | 3.20E-02 | 2.40E-01 | 9.03E+05 | 9.03E+05 | 6.47E+02 | 4.85E+03 | 2.18E+02 | 1.63E+03 | 1.28E+03 | 9.59E+03 |
|  | Ra-228 | 67 | 71.8 | 1.00E-02 | 9.10E-03 | 3.26E-04 | 4.29E-04 | 3.59E-02 | 1.17E+00 | 5.68E-01 | 5.68E+00 | 1.57E+04 | 4.68E+01 |
|  | Th-228 | 140 | 84 | 3.00E-02 | 6.00E-02 | 1.30E+06 | 1.30E+06 | 2.13E+01 | 1.42E+01 | 1.88E+01 | 1.26E+01 | 4.28E+00 | 1.26E+01 |
| Short-lived daughter radionuclides (<10 days λ) | Bi-210 | 44 | 44 | 3.20E-02 | 1.00E-01 | 1.00E+05 | 1.00E+05 | 3.48E-01 | 2.61E+00 | 3.48E-01 | 2.61E+00 | 4.68E+00 | 2.61E+00 |
|  | At-218 | 63 | 63 | 9.10E-03 | 2.00E-02 | 7.00E+01 | 7.00E+01 | 4.13E-11 | 2.72E-10 | 5.65E-11 | 3.73E-10 | 1.88E+01 | 3.73E-10 |
|  | Po-218 | 65 | 65 | 9.10E-03 | 2.40E-01 | 1.36E-04 | 1.36E-04 | 8.29E-07 | 2.53E-05 | 3.73E-02 | 2.46E-01 | 3.48E-01 | 2.05E+00 |
|  | Bi-214 | 140 | 84 | 9.10E-03 | 6.00E-02 | 1.00E+05 | 1.00E+05 | 3.18E-04 | 2.10E-03 | 3.18E-04 | 2.10E-03 | 5.65E-11 | 2.10E-03 |
|  | Pb-214 | 140 | 84 | 9.10E-03 | 6.00E-02 | 9.03E+05 | 9.03E+05 | 1.15E-01 | 7.55E-01 | 3.86E-02 | 2.54E-01 | 3.11E-01 | 1.49E+00 |
|  | Po-214 | 140 | 84 | 9.10E-03 | 6.00E-02 | 1.36E-04 | 1.36E-04 | 7.33E-13 | 2.23E-11 | 3.30E-08 | 2.18E-07 | 3.18E-04 | 1.81E-06 |
|  | Ac-228 | 140 | 84 | 1.00E-02 | 6.00E-02 | 2.00E+06 | 2.00E+06 | 6.27E-02 | 6.27E-01 | 6.21E-02 | 6.21E-01 | 2.26E-01 | 8.39E-01 |
|  | Po-216 | 140 | 84 | 3.00E-02 | 6.00E-02 | 1.36E-04 | 1.36E-04 | 2.13E-09 | 6.57E-09 | 9.60E-05 | 6.40E-05 | 2.75E-07 | 5.34E-04 |
|  | Tl-208 | 44 | 44 | 3.00E-02 | 1.00E-01 | 2.00E+04 | 2.00E+04 | 8.38E-05 | 5.59E-05 | 8.38E-05 | 5.59E-05 | 8.39E-02 | 5.59E-05 |
|  | Bi-212 | 63 | 63 | 3.00E-02 | 2.00E-02 | 1.00E+05 | 1.00E+05 | 3.19E-03 | 2.13E-03 | 3.19E-03 | 2.13E-03 | 8.00E-04 | 2.13E-03 |
|  | Pb-212 | 63 | 63 | 3.00E-02 | 2.00E-02 | 9.03E+05 | 9.03E+05 | 8.87E+00 | 5.91E+00 | 2.99E+00 | 1.99E+00 | 8.38E-05 | 1.17E+01 |
|  | Po-212 | 63 | 63 | 3.00E-02 | 2.00E-02 | 1.36E-04 | 1.36E-04 | 2.82E-15 | 8.68E-15 | 1.27E-10 | 8.45E-11 | 3.19E-03 | 7.05E-10 |
|  | Ra-224 | 63 | 63 | 3.00E-02 | 2.00E-02 | 3.26E-04 | 4.29E-04 | 1.18E-02 | 2.57E-02 | 1.86E-01 | 1.24E-01 | 1.75E+01 | 1.02E+00 |
|  | a Values were retrieved from the 28-d experiment |  |  |  |  |  |  |  |  |  |  |  |  |
|  | b Values are the highest activity concentrations detected in the scale-spiked treatments |  |  |  |  |  |  |  |  |  |  |  |  |
|  | c Values are the highest detected activity concentrations recorded over the 28 d exposure period |  |  |  |  |  |  |  |  |  |  |  |  |
|  | d All activity concentrations of the radionuclides in the overlying seawaters, except for Po-210, where calculated using the scale-specific Kd values |  |  |  |  |  |  |  |  |  |  |  |  |
|  | e Kd values were calculated accordingly; Po-210: using the average seawater activity in each of the scale-spiked treatments divided by the maximum activity concentrations in the sediment; all other radionuclides: scale-specific Kd values for the barite scale retrieved from MacIntosh et al., (2022) |  |  |  |  |  |  |  |  |  |  |  |  |
|  | f Values are the highest activity whole-body concentrations recorded in the exposed amphipods in each respective scale-spiked treatment group |  |  |  |  |  |  |  |  |  |  |  |  |
|  | g Values were calculated from the ERICA default concentration ratios for both reference organisms. See Supporting Table 5 |  |  |  |  |  |  |  |  |  |  |  |  |

Supporting Table 8 Chemical and radiological composition of the pulverised (<30 µm) barite (BaSO4) scale added to the scale-spiked sediments (~ 0.8 g ± 0.01 per benthic microcosm). Total recoverable metal concentrations are presented for the bulk pulverised scale and the nominal concentrations for 0.1% of the bulk scale. Data is presented as mean ± SE to 2 s.f. Sediment Guideline Value (DGV) are obtained from the revised ANZECC/ARMCANZ sediment quality guidelines as per ANZG (2018). A ‘-’ is used where no known sediment guideline values for metals were available

| Metal concentrations in scale |  |  | ANZG DGV* |  |
| --- | --- | --- | --- | --- |
| g/kg (d.w.) | Ba | 564 | - |  |
|  | Ca | 3.7 | - |  |
|  | Fe | 2.9 | - |  |
|  | K | 1.4 | - |  |
|  | Sr | 7.5 | - |  |
|  | S | 158 | - |  |
| mg/kg d.w. | As | <0.6 | - |  |
|  | Cd | 0.2 | 1.5 |  |
|  | Co | 0.5 | - |  |
|  | Cr | 151 | 80 |  |
|  | Cu | 448 | 65 |  |
|  | Hg | 0.28 | 0.15 |  |
|  | Mn | 21.7 | - |  |
|  | Ni | 7.35 | 21 |  |
|  | Pb | 198 | 50 |  |
|  | Zn | 105 | 200 |  |
| Radionuclide activity concentrations in scale |  |  |  | IAEA low-level radioactive waste exemption criteria** |
| 238U decay chain (Bq/g) | 238Ua | <0.007 |  | 1 |
|  | 230Thb | <LOD |  | 1 |
|  | 226Rab | 186 ± 19** |  | 1 |
|  | 210Pbb | 84 ± 8** |  | 1 |
|  | 210Poc | 57 ± 3** |  | 1 |
| 232Th decay chain (Bq/g) | 232Thd | <LOD |  | 1 |
|  | 228Rab | 34 ± 3** |  | 1 |
|  | 228Thb | 49 ± 5** |  | 1 |
| Total contained activity |  | 410 Bq/g |  |  |
| a,d Microwave digest/ICP-MS- solid |  |  |  |  |
| b Gamma spectrometry |  |  |  |  |
| c Alpha spectrometry |  |  |  |  |
| *ANZG (2018) |  |  |  |  |
| ** IAEA low-level radioactive waste exemption criteria is 1 Bq/g (IAEA, 2004) |  |  |  |  |

Supporting Table 9 Activity concentrations of the radionuclides in the sediments after 28-days. All values are expressed in Bq/kg dry weight. Data is presented as the detected value with a SE to 2 s.f., n=5 per treatment. Control is indicative of control sediments with no scale, Scalesurface represents scale added to the surficial layer to a final concentration of 0.1% relative to the sediment mass and Scalemixed represents scale mixed with the sediment to a final concentration of 0.1% relative to the sediment mass. Quality control blank samples were analysed for QA/QC checks. ‘<LOD’ was below the limit of detection.

| Sample ID | Treatment group |
| --- | --- |
|  |  | 210Po activity Bq/kg |  |  | 226Ra activity Bq/kg |  |  |
| Quality control blank sample | Blank |  | < LOD |  |  | < LOD |  |
| AMC1 | Control | 299 | ± | 29 |  | < LOD |  |
| AMC2 |  | 281 | ± | 29 |  | < LOD |  |
| AMC3 |  | 735 | ± | 44 |  | < LOD |  |
| AMC4 |  | 387 | ± | 31 |  | < LOD |  |
| AMC5 |  | 367 | ± | 35 |  | < LOD |  |
| AMS1 | Scalesurface | 66.2 | ± | 3.1 | 184 | ± | 16 |
| AMS2 |  | 71.8 | ± | 2.3 | 156 | ± | 13 |
| AMS3 |  | 60.2 | ± | 2.7 | 154 | ± | 14 |
| AMS4 |  | 31.6 | ± | 2.1 | 62.4 | ± | 6.1 |
| AMS5 |  | 21.1 | ± | 1.4 | 97.6 | ± | 9.1 |
| AMM1 | Scalemixed | 67.3 | ± | 2.9 | 101 | ± | 10 |
| AMM2 |  | 53.5 | ± | 2.7 | 140 | ± | 12 |
| AMM3 |  | 41.3 | ± | 2.1 | 90.5 | ± | 8.4 |
| AMM4 |  | 34.9 | ± | 2.4 | 78.4 | ± | 7.4 |
| AMM5 |  | 40.2 | ± | 2.3 | 105 | ± | 10 |
| Quality control samples | Ctrl (1) | 3.7 | ± | 0.1 |  |  |  |
|  | Ctrl (2) | 4.0 | ± | 0.1 |  |  |  |
|  | Ctrl (3) | 3.8 | ± | 0.1 |  |  |  |
|  | Target activity | 4.12 | ± | 0.15 | 0.020 | ± | 0.002 |

Supporting Table 10 Activity concentrations of the radionuclides in the amphipods (V. australiensis; 3 replicates per sample) after 28-days of exposure. Values are expressed in Bq/kg dry and fresh weight. Control is indicative of control sediments with no scale, Scalesurface represents scale added to the surficial layer to a final concentration of 0.1% relative to the sediment mass and Scalemixed represents scale mixed with the sediment to a final concentration of 0.1% relative to the sediment mass. Quality control blank samples were analysed for QA/QC checks.

| Sample ID | Treatment group | Sample mass (g) | Dry weight (Bq/kg) |  |  |  |  |  | Moisture content of fresh amphipods (%) | Fresh weight (Bq/kg) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 210Po activity Bq/kg |  |  | 226Ra activity Bq/kg |  |  |  | 210Po activity Bq/kg | 226Ra activity Bq/kg |
| Quality control blank sample | Blank | 1 |  | <LOD |  |  | <LOD |  |  |  |  |
| AMC1 | Control | 0.008 | 299 | ± | 29 | 133 | ± | 26 | 81.7 | 55 | 24 |
| AMC2 |  | 0.008 | 281 | ± | 29 |  | <LOD |  | 97.9 | 6 | 0 |
| AMC3 |  | 0.008 | 735 | ± | 44 |  | <LOD |  | 81.7 | 134 | 0 |
| AMC4 |  | 0.009 | 387 | ± | 31 | 60 | ± | 18 | 84.6 | 60 | 9 |
| AMC5 |  | 0.007 | 367 | ± | 35 | 77 | ± | 23 | 82.9 | 63 | 13 |
| AMS1 | Scalesurface | 0.007 | 1856 | ± | 96 | 6578 | ± | 639 | 96.0 | 75 | 265 |
| AMS2 |  | 0.004 | 3818 | ± | 226 | 16106 | ± | 1509 | 89.7 | 393 | 1657 |
| AMS3 |  | 0.004 | 1390 | ± | 101 | 2544 | ± | 271 | 84.3 | 219 | 400 |
| AMS4 |  | 0.000 | 2380 | ± | 1207 | 4957 | ± | 2476 | 99.8 | 5 | 10 |
| AMS5 |  | 0.008 | 1140 | ± | 61 | 2146 | ± | 209 | 86.3 | 157 | 295 |
| AMM1 | Scalemixed | 0.004 | 6106 | ± | 373 | 30960 | ± | 3195 | 99.0 | 63 | 321 |
| AMM2 |  | 0.011 | 719 | ± | 38 | 1432 | ± | 158 | 95.6 | 32 | 63 |
| AMM3 |  | 0.001 | 322 | ± | 132 |  | <LOD |  | 97.0 | 10 | 664 |
| AMM4 |  | 0.010 | 787 | ± | 41 | 2699 | ± | 240 | 96.4 | 28 | 97 |
| AMM5 |  | 0.006 | 1571 | ± | 88 | 3243 | ± | 297 | 94.2 | 91 | 188 |
| Quality control samples | Ctrl (1) |  | 3.7 | ± | 0.1 |  |  |  |  |  |  |
|  | Ctrl (2) |  | 4.0 | ± | 0.1 |  |  |  |  |  |  |
|  | Ctrl (3) |  | 3.8 | ± | 0.1 |  |  |  |  |  |  |
|  | Target activity |  | 4.12 | ± | 0.15 | 0.020 | ± | 0.002 |  |  |  |

Supporting Table 11 Estimated total dose rates (mGy hr-1; 2 significant figures) to the marine organisms used in this study and their respective ERICA reference model organism; V. australiensis (Crustacean), T. deltoidalis (Mollusc-bivalve); N. australiensis (Polychaete worm) for both scale-treatment groups (Mixed and Surface). Parameters used included the respective maximum scale-treatment specific sediment and seawater radionuclide activity concentrations, experimental-specific Kd values and CRs for 226Ra and 210Po. All other radionuclides used scale-specific Kd values from MacIntosh et al., (2022) and default ERICA CRs. Values highlighted in bold exceed the ERICA screening value of 10 mGy/hr and the respective ICRP Derived Consideration Reference Level (DCRL) 400-4000 mGy/hr for Reference Crab (crustaceans and bivalves)

|  | Total dose rate (mGy/hr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Scale-mixed |  |  | Scale-surface |  |  |
| Radionuclide | Crustaceana | Mollusc- bivalveb | Polychaete wormb | Crustaceana | Mollusc- bivalveb | Polychaete wormb |
| Ra-226 | 9.29E+01 | 1.92E-01 | 7.43E-01 | 2.89E-01 | 5.51E-01 | 4.03E+00 |
| Po-218 | 8.77E-07 | 3.04E-03 | 2.53E-02 | 2.55E-06 | 2.01E-02 | 1.67E-01 |
| Pb-214 | 5.31E-03 | 1.79E-03 | 1.05E-02 | 3.50E-02 | 1.18E-02 | 6.89E-02 |
| Bi-214 | 1.92E-05 | 2.03E-05 | 2.16E-05 | 9.96E-05 | 1.00E-04 | 1.01E-04 |
| Po-214 | 3.36E-14 | 1.51E-09 | 1.26E-08 | 1.02E-12 | 9.94E-09 | 8.29E-08 |
| Pb-210 | 1.08E+01 | 3.63E+00 | 2.13E+01 | 8.10E+01 | 2.73E+01 | 1.60E+02 |
| Bi-210 | 9.13E-03 | 9.14E-03 | 9.15E-03 | 6.85E-02 | 6.85E-02 | 6.84E-02 |
| Po-210 | 2.84E+00 | 5.86E+01 | 4.89E+02 | 1.23E+01 | 1.33E+01 | 1.11E+02 |
| Ra-228 | 1.98E-02 | 4.11E-02 | 1.71E-01 | 5.46E-02 | 1.97E-01 | 1.45E+00 |
| Ac-228 | 3.59E-03 | 3.56E-03 | 4.80E-03 | 3.57E-02 | 3.53E-02 | 4.77E-02 |
| Th-228 | 4.00E+00 | 3.55E+00 | 3.56E+00 | 2.68E+00 | 2.38E+00 | 2.39E+00 |
| Ra-224 | 2.37E-03 | 2.98E-02 | 2.41E-01 | 4.55E-03 | 2.01E-02 | 1.61E-01 |
| Po-216 | 4.26E-10 | 8.27E-06 | 6.89E-05 | 8.09E-10 | 5.52E-06 | 4.60E-05 |
| Pb-212 | 4.12E-01 | 1.39E-01 | 8.12E-01 | 2.75E-01 | 9.24E-02 | 5.41E-01 |
| Bi-212 | 1.53E-04 | 1.55E-04 | 1.56E-04 | 1.04E-04 | 1.05E-04 | 1.07E-04 |
| Tl-208 | 2.96E-07 | 3.41E-07 | 3.99E-07 | 2.81E-07 | 3.31E-07 | 3.90E-07 |
| Po-212 | 1.45E-16 | 6.55E-12 | 5.46E-11 | 4.48E-16 | 4.37E-12 | 3.64E-11 |
| At-218 | 4.65E-12 | 6.25E-12 | 6.55E-12 | 2.39E-11 | 3.27E-11 | 3.28E-11 |
| Total dose rate per organism (mGy/hr) | 111 | 66 | 515 | 96 | 44 | 280 |
| a Dose rates were determined using experimental-specific parameters (experimental-calculated Kd values for 226Ra and 210Po only with the remaining radionuclides using default ERICA values; tissue-specific activity concentrations) |  |  |  |  |  |  |
| b Dose rates were determined using experimental-specific parameters (experimental-calculated Kd values for 226Ra and 210Po only with the remaining radionuclides using default ERICA values; ERICA default CR) |  |  |  |  |  |  |

Supporting Table 12 Estimated total external dose rates (mGy hr-1; 2 significant figures) to the marine organisms used in this study and their respective ERICA reference model organism; V. australiensis (Crustacean), T. deltoidalis (Mollusc-bivalve); N. australiensis (Polychaete worm) for both scale-treatment groups (Mixed and Surface). Parameters used included the respective maximum scale-treatment specific sediment and seawater radionuclide activity concentrations, experimental-specific Kd values and CRs for 226Ra and 210Po. All other radionuclides used scale-specific Kd values from MacIntosh et al., (2022) and default ERICA CRs.

|  | External dose rate (mGy/hr) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Scale-mixed |  |  | Scale-surface |  |  |
| Radionuclide | Crustaceana | Mollusc- bivalveb | Polychaete wormb | Crustaceana | Mollusc- bivalveb | Polychaete wormb |
| Ra-226 | 9.40E-02 | 1.20E-01 | 1.45E-01 | 6.72E-13 | 8.56E-13 | 1.04E-12 |
| Po-218 | 8.09E-07 | 1.03E-06 | 1.25E-06 | 2.68E-06 | 3.41E-06 | 4.14E-06 |
| Pb-214 | 6.99E-06 | 8.91E-06 | 1.08E-05 | 5.49E-06 | 6.97E-06 | 8.48E-06 |
| Bi-214 | 4.46E-06 | 5.68E-06 | 6.90E-06 | 4.20E-06 | 5.35E-06 | 6.49E-06 |
| Po-214 | 1.41E-16 | 3.99E-16 | 7.35E-16 | 3.42E-06 | 1.29E-05 | 2.59E-05 |
| Pb-210 | 2.16E-04 | 7.18E-04 | 1.38E-03 | 0.00E+00 | 0.00E+00 | 0.00E+00 |
| Bi-210 | 3.42E-06 | 1.29E-05 | 2.59E-05 | 2.66E-07 | 3.31E-07 | 3.94E-07 |
| Po-210 | 2.49E-07 | 3.09E-07 | 3.67E-07 | 8.46E-17 | 2.39E-16 | 4.41E-16 |
| Ra-228 | 1.88E-02 | 2.38E-02 | 2.88E-02 | 2.42E-10 | 3.07E-10 | 3.73E-10 |
| Ac-228 | 2.27E-05 | 2.87E-05 | 3.48E-05 | 2.27E-05 | 2.87E-05 | 3.48E-05 |
| Th-228 | 3.63E-02 | 4.60E-02 | 5.59E-02 | 2.52E-07 | 3.10E-07 | 3.71E-07 |
| Ra-224 | 5.31E-04 | 6.74E-04 | 8.19E-04 | 5.64E-02 | 7.19E-02 | 8.72E-02 |
| Po-216 | 2.42E-10 | 3.07E-10 | 3.73E-10 | 6.39E-05 | 8.11E-05 | 9.85E-05 |
| Pb-212 | 6.39E-05 | 8.11E-05 | 9.85E-05 | 1.88E-02 | 2.38E-02 | 2.88E-02 |
| Bi-212 | 5.49E-06 | 6.97E-06 | 8.48E-06 | 2.17E-04 | 7.19E-04 | 1.38E-03 |
| Tl-208 | 2.52E-07 | 3.10E-07 | 3.71E-07 | 5.31E-04 | 6.74E-04 | 8.19E-04 |
| Po-212 | 0.00E+00 | 0.00E+00 | 0.00E+00 | 3.63E-02 | 4.60E-02 | 5.59E-02 |
| At-218 | 1.12E-12 | 1.43E-12 | 1.73E-12 | 4.85E-07 | 6.19E-07 | 7.51E-07 |
| Total external dose rate per organism (mGy/hr) | 0.14 | 0.19 | 0.23 | 0.11 | 0.14 | 0.17 |
| a Dose rates were determined using experimental-specific parameters (experimental-calculated Kd values for 226Ra and 210Po only with the remaining radionuclides using default ERICA values; tissue-specific activity concentrations) |  |  |  |  |  |  |
| b Dose rates were determined using experimental-specific parameters (experimental-calculated Kd values for 226Ra and 210Po only with the remaining radionuclides using default ERICA values; ERICA default CR) |  |  |  |  |  |  |