## Supplementary Material 1 – Further sample collection details

At all sample sites, physicochemical parameters including temperature, specific conductance (SC), pH, Eh and dissolved oxygen (DO) were continuously monitored in a flow cell during pumping. Samples were collected once three bore volumes had been purged and/or physicochemical parameters stabilisation was achieved and had been recorded. Unless stated otherwise, all samples were filtered through a 0.45 μm filter prior to collection. With exception of the SO42- and N2O sample collection, all collection bottles/vials were rinsed with the filtered sample water three times prior to filling. Once collected, all samples were sealed with tape to minimise potential atmospheric exchange prior to analysis.

Water samples for anion and δ2H-H2O and δ18O-H2O analyses were stored in 60 mL high-density polyethylene (HDPE) bottles and 30 mL HDPE bottles, respectively. Samples for 14C and 36Cl analysis were collected in 1 L narrow mouth HDPE bottles. Samples for 3H analysis were collected in 2 L HDPE bottles. Samples for cation analysis were collected in 60 mL HDPE bottles and acidified with ultrapure ~70% nitric acid (HNO3). The water for δ13C-DIC was further filtered through a 0.22 μm filter and stored in 12 mL Exetainer vials. The samples outlined above were refrigerated at 4℃.

Samples for δ13C-DOC were filtered through a 0.22 μm filter and stored in a 60 mL HDPE bottle, and frozen within 12 hours of collection. Samples for δ15N-NO3- and δ18O-NO3- were filtered to 0.1 µm and stored in 60 mL HDPE bottles, and frozen within 12 hours of collection.

Prior to dissolved SO42- collection for isotopic analyses, redox conditions (Eh and field HS-) were assessed in the field to avoid samples having undergone significant isotope fractionation. Samples were collected in an in-line closed system with sample filtered through a 0.45 μm high-capacity filter and then passed through an ion-exchange column packed with 22 g of (AmberliteTM IRA458 CL). The resin was conditioned with a 3M KCl solution, prior to field deployment, using methods in Carmody et al. (1998). The volume of groundwater passed through the columns varied and was adapted from a field sulfate turbidimetric analysis (Hanna Instruments). A maximum volume of ~3-10 L was passed through the columns. Sulfate was eluted using a 3M KCl solution upon return to the laboratory. The eluted solution was acidified with 50% (v/v) HCl-ultrapure water to a pH of ~3 to prevent precipitation of carbonates, then dissolved SO42- was fixed as BaSO4 by addition of 1M BaCl2 solution. The BaSO4 was rinsed thoroughly, dried and micronised for subsequent analysis.

## Supplementary Material 2 – Extended EIA dataset

Table S2-1. Extended EIA dataset.

| Sample | Bore ID | Latitude | Longitude | Sampling date | Screen lithology | Screened interval (m) | Screen midpoint (m) | SWL (m) | Head | Alkalinity (mg/L CaCO3) | HCO3 (mg/L) | Temperature (℃) | SC (us/cm3) | TDS (mg/L) | DO (mg/L) | pH | ORP | Fe2+ (mg/L) | Eh (V) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NogGW1 | 13020030 | -23.526392 | 148.141234 | 25/10/2018 | Alluvium | 15.5 - 16.4 | 15.95 | 10.46 | 168.27 | 491 | 599.0 | 28.93 | 4688 | 3152.5 | 0.19 | 6.94 | 45.4 | 0.04 | 0.2454 |
| NogGW2 | 13020089 | -23.522012 | 148.130371 | 25/10/2018 | Tertiary Basalt | 17 - 18 | 17.5 | 6.415 | 175.715 | 758.3 | 925.1 | 25.95 | 3880 | 3021.8 | 0.49 | 6.95 | 95.6 | 0 | 0.2956 |
| NogGW3 | 13020087 | -23.513314 | 148.112325 | 26/10/2018 | Tertiary Basalt | 19.5 - 20.5 | 20 | 9.3 | 190.9 | 565.2 | 689.5 | 25.12 | 1639 | 1383.8 | 0.74 | 7.26 | 57.7 | 0.03 | 0.2577 |
| NogGW4 | 13020106 | -23.51078 | 148.09392 | 26/10/2018 | Tertiary Basalt | 22.3 - 23.3 | 22.8 | 12.63 | 194.16 | 343.9 | 419.6 | 27.35 | 739 | 648.9 | 3.31 | 7.31 | 112.5 | 0.01 | 0.3125 |
| NogGW5 | 13020086 | -23.508189 | 148.070748 | 26/10/2018 | Tertiary Basalt | 29.1 - 30.1 | 29.6 | 17.69 | 195.21 | 483.2 | 589.5 | 27.47 | 2372 | 1652.9 | 4.43 | 6.89 | 83.2 | 0 | 0.2832 |
| NogGW6 | 13020090 | -23.497287 | 148.056347 | 27/10/2018 | Tertiary Basalt | 16.5 - 17.5 | 17 | 4.77 | 185.73 | 430.7 | 525.5 | 27.06 | 1908 | 1541.5 | 1.4 | 7.29 | 108.9 | 0 | 0.3089 |
| NogGW7 | 13020107 | -23.492198 | 148.072989 | 27/10/2018 | Tertiary Basalt | 8.8 - 9.8 | 9.3 | 3.23 | 192.84 | 379.2 | 462.6 | 24.4 | 1337 | 1061.1 | 5.13 | 7.05 | 111.4 | 0 | 0.3114 |
| NogGW8 | 13020096 | -23.474511 | 148.079923 | 27/10/2018 | Tertiary Basalt | 8.6 - 9.6 | 9.1 | 2.16 | 191.63 | 284.4 | 347.0 | 25.87 | 668 | 568.0 | 0.46 | 7.7 | 41.8 | 0.01 | 0.2418 |
| NogGW9 | 13020108 | -23.464976 | 148.114916 | 27/10/2018 | Tertiary Basalt | 14.5 - 15.5 | 15 | 7.32 | 187.22 | 180.2 | 219.8 | 27.34 | 454 | 436.7 | 0.24 | 6.37 | 28.1 | 0.05 | 0.2281 |
| NogGW10 | 13020041 | -23.534435 | 148.083898 | 28/10/2018 | Tertiary Basalt | 5.9 - 6.9 | 6.4 | 3.19 | 192.97 | 206.3 | 251.7 | 23.95 | 445 | 394.7 | 0.4 | 7.55 | 88.9 | 0 | 0.2889 |
| NogGW11 | 13020114A | -23.573472 | 148.212586 | 28/10/2018 | Tertiary Basalt | 27.15 - 29.85 | 28.5 | 10.23 | 178.23 | 685.8 | 836.7 | 26.44 | 4191 | 3054.1 | 0.24 | 6.87 | 62.7 | 0 | 0.2627 |
| NogGW12 | 13020116 | -23.565155 | 148.214809 | 28/10/2018 | Alluvium | 11.8 - 15 | 13.4 | 10.03 | 173.71 | 118.9 | 145.1 | 27.15 | 1855 | 1136.4 | 1.3 | 5.95 | 98 | 0 | 0.2980 |
| NogGW13 | 13020165 | -23.437927 | 148.330536 | 28/10/2018 | Alluvium | 13.2 - 15 | 14.1 | 13.67 | 147.65 | 648.4 | 791.0 | 28.18 | 5274 | 3371.9 | 0.54 | 6.83 | -229.8 | 0.28 | -0.0298 |
| NogGW14 | 13020035 | -23.441963 | 148.147643 | 29/10/2018 | Alluvium | 14.3 - 15.2 | 14.75 | 10.44 | 164.66 | 426.1 | 519.8 | 27.43 | 5138 | 3379.2 | 2.44 | 7.13 | 133.2 | 0 | 0.3332 |
| NogGW15 | 13020111 | -23.466321 | 148.155576 | 29/10/2018 | Tertiary Basalt | 28.6 - 29.6 | 29.1 | 20.73 | 166.74 | 238.5 | 291.0 | 29.96 | 589 | 488.7 | 0.34 | 7.54 | -91.9 | 0 | 0.1081 |
| NogGW16 | 13020207 | -23.579372 | 148.087368 | 29/10/2018 | Aldeberan Sandstone | 33.4 - 39.4 | 36.4 | 11.44 | 182.07 | 226.9 | 276.8 | 25.66 | 2472 | 1554.0 | 0.12 | 6.39 | -21.9 | 2.93 | 0.1781 |
| NogGW17 | 13020208 | -23.383363 | 148.187026 | 30/10/2018 | Tertiary Sandstone | 94.3 - 96.3 | 95.3 | 41.06 | 156.5 | 423.4 | 516.5 | 28.24 | 1377 | 1096.2 | 0.18 | 8.2 | 57.7 | 0 | 0.2577 |
| NogGW18 | 13020032A | -23.489577 | 148.150822 | 30/10/2018 | Tertiary Basalt | 23.1 - 24 | 23.55 | 4.95 | 172.42 | 329.1 | 401.5 | 24.5 | 1640 | 1190.1 | 0.96 | 7.12 | -116.2 | 0 | 0.0838 |
| NogGW19 | 13020092 | -23.469339 | 148.082512 | 31/10/2018 | Tertiary Basalt | 29.1 - 30.1 | 29.6 | 3.13 | 182.16 | 703.5 | 858.3 | 26.12 | 2145 | 1734.1 | 2.61 | 7.1 | 23.8 | 0 | 0.2238 |
| NogGW20 | 13020072 | -23.497912 | 148.084566 | 31/10/2018 | Tertiary Basalt | 11.7 - 12.7 | 12.2 | 8.83 | 193.44 | 349.9 | 426.9 | 25.38 | 1300 | 979.1 | 4.05 | 6.98 | 57.2 | 0 | 0.2572 |
| NogGW21 | 90322 | -23.51648 | 148.163423 | 31/10/2018 | Alluvium | 18.5 - 22 | 20.25 | n.d. | n.d. | 755.6 | 921.8 | 26.41 | 5285 | 3746.5 | 0.37 | 6.98 | 75.9 | 0 | 0.2759 |
| NogGW22 | 13020101 | -23.538764 | 148.115059 | 1/11/2018 | Tertiary Basalt | 17.6 - 18.6 | 18.1 | 5.61 | 180.17 | 512.9 | 625.7 | 24.65 | 2329 | 1716.8 | 0.98 | 6.99 | 56.7 | 0.04 | 0.2567 |
| NogGW23 | Private Bore | -23.378087 | 148.306553 | 1/11/2018 | Unknown | n.d. | n.d | n.d. | n.d. | 526.7 | 642.6 | 26.05 | 1214 | 1033.0 | 0.2 | 7.06 | 39.9 | 0 | 0.2399 |
| NogGW24 | 13020117A | -23.50732 | 148.265605 | 1/11/2018 | Alluvium | 14 - 17 | 15.5 | 12.97 | 159.42 | 25.8 | 31.5 | 27.6 | 719 | 520.5 | 6.43 | 5.52 | 151.1 | 0.05 | 0.3511 |
| NogSW1-04 | N/A | -23.658105 | 148.074308 | 29/10/2018 | N/A | N/A | N/A | N/A | N/A | 138.6 | 169.1 | 30.62 | 356 | 231 | 9.05 | 8.27 | 67.1 | 0 | 0.27 |
| NogSW2-04 | N/A | -23.531183 | 148.167295 | 29/10/2018 | N/A | N/A | N/A | N/A | N/A | 145 | 176.9 | 28.19 | 420 | 273 | 5.61 | 7.58 | 74.5 | 0 | 0.27 |
| NogSW3-04 | N/A | -23.5131 | 148.207067 | 30/10/2018 | N/A | N/A | N/A | N/A | N/A | 134.7 | 164.3 | 28.34 | 462 | 300 | 6.22 | 7.76 | 92.7 | 0 | 0.29 |
| NogSW5-04 | N/A | -23.432029 | 148.331279 | 30/10/2018 | N/A | N/A | N/A | N/A | N/A | 141.9 | 173.1 | 27.1 | 472 | 307 | 5.75 | 7.82 | 112.4 | 0 | 0.31 |

n.d. not determined (sample concentration below quantification limit).

N/A not applicable.

Table S2-1. Extended EIA dataset (continued).

| Sample | Bore ID | Water-type | F- (mg/L) | Cl-(mg/L) | SO42-(mg/L) | Br-(mg/L) | PO43-(mg/L) | Ca (mg/L) | Fe (mg/L) | K (mg/L) | Li (mg/L) | Mg (mg/L) | Na (mg/L) | P (mg/L) | S (mg/L) | Si (mg/L) | Ba (µg/L) | Mn (µg/L) | Sr (µg/L) | DOC (mg/L) | NO2-(mg/L N) | NO3- (mg/L N) | NH3-N (mg/L N) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NogGW1 | 13020030 | Na-Cl | 0.39 | 1250 | 290 | 2.79 | <1 | 75.8 | <0.005 | 6.19 | 0.023 | 323 | 430 | 0.08 | 102 | 29.5 | 109 | 308 | 1430 | 1.53 | <0.02 | 9.46 | 0.000 |
| NogGW2 | 13020089 | Na-HCO3 | 0.21 | 766 | 396 | 1.69 | <1 | 93.8 | <0.005 | 5.9 | 0.017 | 279 | 367 | 0.04 | 138 | 26.8 | 15 | <1 | 1410 | 1.10 | <0.02 | 4.70 | 0.000 |
| NogGW3 | 13020087 | Na-HCO3 | 0.12 | 199 | 63.8 | 0.58 | <1 | 42 | 0.02 | 2.26 | 0.006 | 80.6 | 216 | 0.04 | 24 | 23.4 | 29 | <1 | 853 | 0.81 | <0.02 | 9.37 | 0.000 |
| NogGW4 | 13020106 | Na-HCO3 | 0.53 | 28.1 | 12.1 | 0.16 | <1 | 23.2 | <0.005 | 0.99 | 0.002 | 41 | 82.2 | 0.06 | 4.7 | 27.9 | 3 | 9 | 265 | 0.77 | <0.02 | 1.83 | 0.000 |
| NogGW5 | 13020086 | Na-HCO3 | 0.04 | 476 | 61.4 | 1.44 | <1 | 100 | <0.005 | 4.55 | 0.005 | 136 | 176 | 0.03 | 23.2 | 26.3 | 21 | <1 | 1270 | 0.57 | <0.02 | 12.90 | 0.000 |
| NogGW6 | 13020090 | Na-HCO3 | 0.31 | 200 | 252 | 1.13 | <1 | 33.6 | <0.005 | 0.82 | 0.019 | 43.9 | 309 | 0.01 | 95.1 | 20.6 | 17 | 4 | 515 | 0.72 | <0.02 | 13.32 | 0.000 |
| NogGW7 | 13020107 | Ca-HCO3 | 0.14 | 187 | 46.6 | 0.84 | <1 | 107 | <0.005 | 0.83 | 0.005 | 75.6 | 72 | 0.01 | 17.1 | 26 | 9 | <1 | 849 | 1.08 | <0.02 | 14.57 | 0.000 |
| NogGW8 | 13020096 | Na-HCO3 | 0.45 | 35.9 | 7.11 | 0.2 | <1 | 16.9 | <0.005 | 0.06 | 0.004 | 20.7 | 95.9 | 0.01 | 2.8 | 35.4 | 7 | <1 | 273 | 0.64 | <0.02 | 1.20 | 0.000 |
| NogGW9 | 13020108 | Na-HCO3 | 0.29 | 38.8 | 14.2 | 0.23 | <1 | 3.66 | 0.067 | 0.27 | 0.005 | 4.16 | 107 | 0.5 | 5.4 | 42.1 | 9 | 119 | 51 | 2.06 | <0.02 | <0.02 | 0.000 |
| NogGW10 | 13020041 | Ca-HCO3 | 0.3 | 25.4 | 5.83 | 0.18 | <1 | 30.8 | <0.005 | 0.35 | 0.004 | 28 | 27.3 | 0.03 | 2.3 | 21.3 | 3 | 2 | 308 | 2.14 | <0.02 | 0.20 | 0.000 |
| NogGW11 | 13020114A | Na-Cl | 0.22 | 1010 | 184 | 2.43 | <1 | 90.3 | <0.005 | 7.96 | 0.054 | 128 | 679 | 0.06 | 72.6 | 33.8 | 57 | <1 | 1320 | 1.13 | <0.02 | 1.72 | 0.000 |
| NogGW12 | 13020116 | Na-Cl | 0.08 | 479 | 30.5 | 1.21 | <1 | 16.2 | 0.348 | 1.8 | 0.043 | 22.2 | 307 | 0.7 | 11.8 | 46.7 | 45 | 24 | 272 | 3.25 | <0.02 | 16.44 | 0.009 |
| NogGW13 | 13020165 | Na-Cl | 0.31 | 1390 | 95.6 | 3.9 | <1 | 225 | 0.292 | 3.56 | 0.04 | 214 | 572 | 0.3 | 38 | 34.2 | 124 | 690 | 2790 | 1.46 | <0.02 | <0.02 | 0.000 |
| NogGW14 | 13020035 | Na-Cl | 0.36 | 1450 | 184 | 2.87 | <1 | 81.6 | 0.011 | 4.71 | 0.049 | 171 | 824 | 0.2 | 73.3 | 37.9 | 42 | 9 | 1580 | 1.54 | <0.02 | 6.26 | 0.000 |
| NogGW15 | 13020111 | Na-HCO3 | 0.33 | 45.5 | 6.85 | 0.22 | <1 | 22.2 | 0.023 | 0.8 | 0.003 | 39.7 | 54.1 | 0.06 | 2.7 | 23.8 | 12 | 36 | 276 | 2.62 | 0.03 | 0.23 | 0.009 |
| NogGW16 | 13020207 | Na-Cl | 0.16 | 639 | 112 | 1.89 | <1 | 99.5 | 3.09 | 26.5 | 0.082 | 111 | 234 | 0.03 | 41.4 | 7.4 | 99 | 102 | 946 | 0.42 | <0.02 | <0.02 | 0.112 |
| NogGW17 | 13020208 | Na-HCO3 | 0.12 | 165 | 68.5 | 0.56 | <1 | 6.31 | 0.006 | 6.02 | 0.049 | 8.19 | 285 | 0.07 | 26.1 | 13.5 | 53 | 5 | 197 | 1.95 | <0.02 | <0.02 | 0.335 |
| NogGW18 | 13020032A | Na-HCO3 | 0.29 | 303 | 76.5 | 1.13 | <1 | 68.1 | 0.15 | 0.84 | 0.014 | 111 | 122 | 0.02 | 28.1 | 30.6 | 24 | 19 | 917 | 1.13 | 0.02 | 10.34 | 0.000 |
| NogGW19 | 13020092 | Na-HCO3 | 0.15 | 224 | 42.4 | 0.99 | <1 | 25.7 | 0.011 | 5.08 | 0.025 | 52.1 | 380 | 0.08 | 16.9 | 28.9 | 50 | <1 | 470 | 1.21 | <0.02 | 22.36 | 0.000 |
| NogGW20 | 13020072 | Na-HCO3 | 0.23 | 163 | 22.5 | 0.89 | <1 | 66.7 | 0.008 | 0.46 | 0.004 | 69.9 | 99.1 | 0.03 | 8.5 | 31.9 | 1 | <1 | 1000 | 0.67 | <0.02 | 19.87 | 0.000 |
| NogGW21 | 90322 | Na-Cl | 0.23 | 1090 | 424 | 2.65 | <1 | 102 | <0.005 | 5.76 | 0.062 | 198 | 804 | 0.2 | 147 | 30.2 | 85 | 105 | 1930 | 1.07 | <0.02 | 4.16 | 0.000 |
| NogGW22 | 13020101 | Na-HCO3 | 0.24 | 383 | 116 | 1.2 | <1 | 74.2 | 0.01 | 5.85 | 0.02 | 127 | 251 | 0.04 | 44.3 | 30.4 | 22 | 4 | 823 | 1.22 | <0.02 | 12.87 | 0.000 |
| NogGW23 | Private Bore | Na-HCO3 | 0.24 | 101 | 10.5 | 0.39 | <1 | 43.9 | 0.007 | 4.57 | 0.012 | 79.7 | 117 | 0.04 | 4.3 | 28.3 | 23 | 3 | 490 | 0.37 | <0.02 | <0.02 | 0.000 |
| NogGW24 | 13020117A | Na-Cl | 0.06 | 88.3 | 84.1 | 0.35 | <1 | 10.5 | 0.023 | 5.82 | 0.035 | 14.2 | 102 | 0.04 | 29.1 | 46.3 | 35 | 3 | 188 | 1.27 | <0.02 | 24.39 | 0.000 |
| NogSW1-04 | N/A | Na-HCO3 | 0.28 | 21.6 | 5.34 | 0.15 | <1 | 26.1 | <0.005 | 9.05 | 0.002 | 11.9 | 26.2 | 0.09 | 2.1 | 4 | 54 | <1 | 277 | 5.2 | <0.02 | <0.02 | 0.000 |
| NogSW2-04 | N/A | Na-HCO3 | 0.29 | 36.7 | 8.86 | 0.18 | <1 | 27.4 | <0.005 | 9.06 | 0.002 | 13.9 | 34.9 | 0.1 | 3.5 | 3.9 | 42 | 27 | 298 | 5.2 | <0.02 | <0.02 | 0.025 |
| NogSW3-04 | N/A | Na-HCO3 | 0.28 | 48.2 | 12.2 | 0.21 | <1 | 28 | <0.005 | 9.13 | 0.002 | 14.6 | 40 | 0.1 | 4.6 | 3.9 | 45 | 7 | 313 | 5.4 | <0.02 | <0.02 | 0.022 |
| NogSW5-04 | N/A | Na-HCO3 | 0.28 | 51.3 | 12.9 | 0.22 | <1 | 28.4 | <0.005 | 9.15 | 0.002 | 15.3 | 40.7 | 0.1 | 5 | 3.9 | 59 | 5 | 327 | 5.5 | <0.02 | <0.02 | 0.002 |

n.d. not determined (sample concentration below quantification limit).

N/A not applicable.

Table S2-1. Extended EIA dataset (continued).

| Sample | Bore ID | Water-type | δ2H-H2O (‰) | δ18O-H2O (‰) | δ13C-DIC (‰) | δ13C-DOC (‰) | δ15N-NO3- (‰) | δ18O-NO3- (‰) | δ34S-SO42- (‰) | δ18O-SO42- (‰) | 3H (TU) | 3H age (years BP) | 36Cl/Cl (×10-15) | DENa) (%) | Groundwater cluster |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NogGW1 | 13020030 | Na-Cl | -14.3 | -1.8 | -12.8 | -27.1 | +12.4 | +13.2 | +10.6 | +13.1 | 0.48 | <40-50 | 81.3 | 80.6 | Mixed |
| NogGW2 | 13020089 | Na-HCO3 | -20.5 | -2.9 | -12.7 | -26.8 | +9.9 | +7.5 | +6.8 | +11.9 | 0.22 | <40-50 | 77.5 | 71.3 | Mixed |
| NogGW3 | 13020087 | Na-HCO3 | -8.9 | -1.0 | -11.8 | -25.8 | +7.2 | +7.0 | +8.8 | +13.8 | 0.49 | <40-50 | 74.3 | 56.3 | Mixed |
| NogGW4 | 13020106 | Na-HCO3 | -11.8 | -1.3 | -9.7 | -26.0 | +5.1 | +4.2 | +8.8 | +15.4 | 0.85 | <40-50 | 156.2 | 0.0 | Leakage-dominated |
| NogGW5 | 13020086 | Na-HCO3 | -22.2 | -3.0 | -10.7 | -26.4 | +5.8 | +4.6 | +11.6 | +12 | 0.16 | <40-50 | 76.5 | 45.6 | Mixed |
| NogGW6 | 13020090 | Na-HCO3 | -10.6 | -1.1 | -9.7 | -25.5 | +16.8 | +16.1 | +15.2 | +13.2 | 0.42 | <40-50 | 146.5 | 90.3 | Mixed |
| NogGW7 | 13020107 | Ca-HCO3 | -17.9 | -2.5 | -10.9 | -17.2 | +3.5 | +5.2 | +6.4 | +17.6 | 0.43 | <40-50 | 106.1 | 22.1 | Mixed |
| NogGW8 | 13020096 | Na-HCO3 | -5.7 | -0.3 | -9.4 | -26.3 | +10.3 | +9.7 | +7.0 | +16.1 | 1.10 | <40-50 | 142.2 | 57.4 | Leakage-dominated |
| NogGW9 | 13020108 | Na-HCO3 | -5.5 | -0.2 | -13.1 | -25.6 | n.d. | n.d. | +4.7 | +16.7 | 0.90 | <40-50 | 170.0 | N/A | Leakage-dominated |
| NogGW10 | 13020041 | Ca-HCO3 | +0.9 | +1.2 | -8.6 | -24.6 | +19.6 | +24.8 | +7.6 | +19.0 | 1.55 | <40-50 | 163.0 | 90.7 | Leakage-dominated |
| NogGW11 | 13020114A | Na-Cl | -14.6 | -1.9 | -12.5 | -26.7 | +17.0 | +15.4 | +15.2 | +9.6 | 0.42 | <40-50 | 79.1 | 90.6 | Mixed |
| NogGW12 | 13020116 | Na-Cl | -7.1 | -0.8 | -14.5 | -26.7 | +6.2 | +7.7 | n.d. | n.d. | 1.11 | <40-50 | 91.4 | 48.9 | Mixed |
| NogGW13 | 13020165 | Na-Cl | -13.1 | -1.9 | -13.6 | -23.4 | n.d. | n.d. | +3.9 | +12.1 | 0.59 | <40-50 | 72.8 | N/A | Mixed |
| NogGW14 | 13020035 | Na-Cl | -9.5 | -1.1 | -11.8 | -29.7 | +14.1 | +11.5 | +10.9 | +13.1 | 0.50 | <40-50 | 81.2 | 85.1 | Mixed |
| NogGW15 | 13020111 | Na-HCO3 | -0.4 | +0.8 | -9.8 | -20.3 | +18.8 | +20.3 | n.d. | n.d. | 1.42 | <40-50 | 152.5 | 89.4 | Leakage-dominated |
| NogGW16 | 13020207 | Na-Cl | -31.9 | -5.1 | -14.6 | -27.4 | n.d. | n.d. | +9.0 | +11.6 | 0.05 | >40-50 | 79.8 | N/A | Pre-irrigation |
| NogGW17 | 13020208 | Na-HCO3 | -28.6 | -4.6 | -9.0 | -32.6 | n.d. | n.d. | n.d. | n.d. | 0.06 | >40-50 | 72.8 | N/A | Pre-irrigation |
| NogGW18 | 13020032A | Na-HCO3 | -15.5 | -2.0 | -9.4 | -26.3 | +6.8 | +8.0 | +11.0 | +13.8 | 0.67 | <40-50 | 86.9 | 53.5 | Mixed |
| NogGW19 | 13020092 | Na-HCO3 | -15.3 | -2.0 | -12.6 | -26.1 | +10.2 | +10.1 | n.d. | n.d. | 0.34 | <40-50 | 107.0 | 72.7 | Mixed |
| NogGW20 | 13020072 | Na-HCO3 | -16.7 | -2.3 | -9.7 | -25.8 | +3.9 | +6.1 | n.d. | n.d. | 0.52 | <40-50 | 109.8 | 26.8 | Mixed |
| NogGW21 | 90322 | Na-Cl | -18.4 | -2.7 | -12.3 | -25.9 | +18.5 | +14.3 | +6.7 | +11.3 | 0.54 | <40-50 | 86.4 | 92.5 | Mixed |
| NogGW22 | 13020101 | Na-HCO3 | -14.1 | -1.9 | -11.5 | -28.6 | +6.0 | +5.5 | +11.7 | +13.1 | 0.53 | <40-50 | 86.6 | 47.3 | Mixed |
| NogGW23 | Private Bore | Na-HCO3 | -34.4 | -5.2 | -8.6 | -26.8 | n.d. | n.d. | n.d. | n.d. | 0.10 | >40-50 | 90.8 | N/A | Pre-irrigation |
| NogGW24 | 13020117A | Na-Cl | -15.3 | -2.3 | -15.2 | -25.1 | +1.9 | +5.0 | n.d. | n.d. | 0.99 | <40-50 | 116.3 | 0.0 | Mixed |
| NogSW1-04 | N/A | Na-HCO3 | +11.3 | +2.9 | -3.5 | -24.9 | n.d. | n.d. | n.d. | n.d. | 1.82 | Modern | 159.2 | N/A | N/A |
| NogSW2-04 | N/A | Na-HCO3 | +9.6 | +2.5 | -5.7 | -24.1 | n.d. | n.d. | n.d. | n.d. | 1.99 | Modern | 140.2 | N/A | N/A |
| NogSW3-04 | N/A | Na-HCO3 | +8.0 | +2.2 | -6.0 | -24.6 | n.d. | n.d. | n.d. | n.d. | 1.96 | Modern | 122.8 | N/A | N/A |
| NogSW5-04 | N/A | Na-HCO3 | +7.6 | +2.2 | -6.6 | -24.5 | n.d. | n.d. | n.d. | n.d. | 1.93 | Modern | 109.6 | N/A | N/A |

a) Calculated using Equation 2.

n.d. not determined (sample concentration below quantification limit).

N/A not applicable.

## Supplementary Material 3 – Hierarchical cluster analysis

R software (R Core Team, 2017) was used to determine which hierarchical clustering method was best suited to measure similarity between groundwater samples. This approach has been outlined in https://uc-r.github.io/hc_clustering. The variables of δ2H-H2O, δ18O-H2O, δ13C-DIC, 3H, 36Cl/Cl and NO3- were used in the method, as these were interpreted to best reflect groundwater residence times, recharge sources and distribution of NO3-.

Ward’s minimum variance method, which uses the squared Euclidean distances as a measure of similarity, gives the best clustering structure within the EIA groundwater dataset. Similar applications of Ward’s method for clustering to hydrogeochemical data can be found in Lambrakis et al. (2004) and Swanson et al. (2001). The derived dendogram, which depicts the cluster structures, is shown in Fig. S3-1.

Fig. S3-1. Dendogram derived from Ward’s minimum variance method of hierarchical clustering. The different groundwater clusters are indicated.

## Supplementary Material 4 – Supporting evidence for EIA groundwater hydrogeochemical processes

Figs. S4-1 to S4-3 are used as supporting evidence in this study.

Fig. S4-1. Relationship between NO3- and HCO3- (a) and δ15N-NO3- and HCO3- (b) in the mixed groundwater cluster.

Fig. S4-2. Relationship between NO3- and HCO3- (a) and δ15N-NO3- and HCO3- (b) in the leakage-dominated groundwater cluster.

Fig. S4-3. Relationship between δ15N-NO3- and DOC (a) and δ15N-NO3- and δ13C-DOC (b) in the mixed and leakage-dominated groundwater clusters.

REFERENCES

Carmody, R.W., Plummer, N., Busenberg, E. and Coplen, T.B., 1998. Methods for collection of dissolved sulfate and sulfide and analysis of their sulfur isotopic composition (No. 97-234). US Geological Survey.

R Core Team.: R: A language and environment for statistical computing. R Foundation for Statistical Computing, Vienna, Austria. URL https://www.R-project.org/, 2017.